## Building & deployment of the SE290A_BackOfficeSim on the EBS-50

### Building the application

- Use Visual studio 2022 or higher to open the ‘SE290A_BackOfficeSim_CoreApi.sln’ file

- To publish the project for the EBS-50 use the following publishing configuration:

- Locate the ‘bin\Release\backofficesim_ebs50’ folder

### Deployment

Open WinSCP and open an SFTP connection with the following connection settings:

Host: IP / port:22 / user: root / pass: admin

TIP: If you use RNDIS by micro-usb, the host IP will always be 192.168.100.5, making it easy to recycle the same connection for each EBS-50.

NOTE: It will probably warn you if you want to trust the SSH key that encrypts the connection.

- Create a folder named ‘SE290A_BackOfficeSim’ in '/home/root'

- Copy the content of the ‘backofficesim_ebs50’ folder into this folder

- Copy and replace the files from the ‘LinuxArm’ folder to the same ‘SE290A_BackOfficeSim’ folder to resolve SQLite issues

- Right click the file and change access rights of ‘SE290A_BackOfficeSim_CoreApi' to '0755'

- Right click the file and change access rights of ‘ebs-50.pfx' to '0600'

### Installation of the service

- Browse to: /etc/systemd/system

- Copy file ' SE290A_BackOfficeSim.service' to '/etc/systemd/system'

- Right click the file Change access rights of ‘SE290A_BackOfficeSim.service' to '0755'

To enable and start the service type:

systemctl enable SE290A_BackOfficeSim

systemctl start SE290A_BackOfficeSim

Check that the application is running at:

https://ebs50.local:8794 / http://ebs50.local:8081

### ESL Web Server / EBS-50 configuration files

Depending on the platform (PC or EBS-50). Locate the demo configuration files for the ESL Web Server.

Login on the ESL Web Server / EBS-50 (default : admin/admin)

Import the file under Advanced > Backup / Restore

Login again, but now with: admin / 12345678

## Event handlers

https://localhost:8794 POST

api/%MAC%/%ID%/%EVENT%/%VALUE%/%SEQNR%/%0%/%DESCRIPTION%

or

api/%MAC%/%ID%/%EVENT%/%VALUE%/%SEQNR%/%DESCRIPTION%

%VALUE% contains image status information and current page on the ESL

%0% contains the current ‘Action’ column of the product database (optional)

(Active = ON)

https://localhost:8794 GET

api/KEEP-ALIVE

This reduces latency by keeping the connection alive and kicking